Messaging & Collaboration Programme

Lync System

Functional Requirements Specification

Workstream: Messaging

Author			: Ally Ward

Document Status	: DRAFT

Version			: V2.0

Issue Date		: 14th August 20112

Security Classification	: Internal Use only

- Revision history

| Version | Name | Revision description | Date |
| --- | --- | --- | --- |
| 0.1 | Ally Ward | Initial Draft | 27 July 2012 |
| 0.2 | Ally Ward | Updated with comments | 7 August 2012 |
| 0.3 | Ally Ward | Updated reviewers and amended trace to match ALM tool | 14 August 2012 |
| 0.4 | Ally Ward | Updated from DA review comments Will Phillips, Jan Willem Jansen | 16 August 2012 |
| 0.5 | Ally Ward | Updated from Da review comments Juan Torres | 20 August 2012 |
| 1.0 | Ally Ward | Approved by DA – comments left in to be removed after next version to include voice and other Phase 1 requirements | 29 August 2012 |
| 1.1 | Ally Ward | Additional requirements added for Security logging, Account provisioning, video and voice | 05 September 2012 |
| 2.0 | Ally Ward | Addressed DA comments – approved and up-issued to Version 2 | 04 October 2012 |

- Approvers

| Role | Name | Date accepted |
| --- | --- | --- |
| Lync Programme Manager | Neil Sahdev |  |
| Lync Project Manager | Stephen McCartney |  |
| Enterprise Architecture | Juan Torres | 16 August 2012 |
| InfoSec | Klara Neginsky | 16 August 2012 |
| Infrastructure Engineering | Nick Wood, Brendan Millard | 16 August 2012 |
| Operations | Jan Willem Jansen, Jeroen Schoemaker | 16 August 2012 |
| Solution Architecture | Jim Kao, Kipela Rashidi | 16 August 2012 |
| Lync Lead BA | Ally Ward | 16 August 2012 |

- Reviewers

| Role | Name | Date accepted |
| --- | --- | --- |
| Domino & Messaging Engineer | Chris Rae | 7 August 2012 |
| Lync Design and Build Project Manager | Stephen McCartney |  |
| Lync QA Lead | Tim Brinkman | 7 August 2012 |
| Video Engineer | Joey Weisenstein, | 18 August 2012 |
| Global Lead for Voice & Video Operations | Jan Willem Jansen | 16 August 2012 |
| Voice and Video Engineer | Sandra Cevallos | 16 August 2012 |

- Distribution

| Role | Name | Date accepted |
| --- | --- | --- |
| Business Deployment? | Martin Kremmer |  |
| Business Change / Governance Group | Rachel Phillips |  |
| Q&RM | Steven Jelf |  |
| Infrastructure & Operations | Harold Klein / Jeroen Schoemaker |  |

Introduction and Background	5

Scope exclusions	5

Assumptions	5

Requirement traceability	6

Phase 1 Vision and Scope requirements	6

Business Requirements	6

Operational Requirements	7

Security Requirements	8

System Requirements	9

Unified Messaging Requirements	11

User requirements	11

Phase 1 Requirements to follow	13

Phase 2	14

Business Requirements	14

Operational Requirements	14

System Requirements	14

Unified Messaging Requirements	15

User Requirements	16

Phase 3 – all requirements to follow	17

Requirement descriptions	19

Co-Existence	19

Instant Messaging	20

Remote Access	26

Security	26

System Administration	28

Unified Messaging	29

Video Conferencing	30

Voice Telephony	31

Web Conferencing	33

Non-Functional	35

Functional specifications	39

Design elements (completed with Application Engineering)	39

Dependencies	39

Risks	39

Open items	40

Introduction and Background

This document specifies the requirements for Lync Phase 1.

Out of scope for this document are mobile phone clients, Unified Messaging, Federated Video and Voice, Video integration with EY systems such as Tandberg.

All requirements have been uploaded into the Application Lifecycle Management (ALM )  requirements repository and can be located using their Name or ALM Requirement ID.  The requirements have been categorized into 2 releases within each phase in ALM for testing purposes:

- Ernst and Young Specific Requirements or overlap to Ernst and Young specific development effort – will be proven first in the QA environment and thereafter in the pilot environment

- Configuration based Requirements that need to be tested to ensure that the product is correctly configured or standard Lync functionality expected to work out of the box – Will be proven in the pilot environment first.

This requirements specification is based on Ernst and Young’s standard Template.  The sections covering use case and user interface design have not been included as project activity is focused on configuration rather than software development.  In these circumstances the User Case and user interface specification add little or no value to the specification of the requirements.

Requirements have been allocated a unique name to assist with grouping and tracing requirements. The convention is LY-XXX-YYY-NNN where XXX relates to the functional area for the requirement, YYY is an optional functional sub-area and NNN is a number 001 to 999, unique within sub-area.  This is in addition to the system generated, unique ALM Requirement ID.

E.g. LY-CE-EXCHOUT-001 is Co-existence, Exchange outlook, req 1

Where the requirement name reads LY-MMM-PARM-NNN this indicates a requirement has been derived from configuration parameter settings for a particular functional area. E.g. LY-VC-PARM-002 Disable Full screen Video refers to a client  configuration setting for video conferencing behaviour.

Scope exclusions

The following items have been determined to be out-of-scope for this specification:

Details of some of the security related requirements which will be subject of a separate Infosec document.

Blackberry, Android and Apple mobile phone clients are not discussed in this document; If they come into scope they will be covered in a later version of this document

Unified messaging, Federated Video and Voice and Video integration with EY systems such as Tandberg will be delivered in phase 2 so whilst some requirements for these functions have been captured in the course of determining Phase 1 requirements, detailed requirements will be determined for Phase 2 of Lync.

Assumptions

The following assumptions are associated with the specification of this feature:

The requirements described are not intended as an exhaustive functional description of the applications because it is assumed that Microsoft Lync is a mature application that operates successfully.  Instead it is the intention of this document to identify configuration choices and custom Ernst and Young requirements that need to be tested.  In addition, requirements describing out of the box functionality have been included where they put the testable requirements into context.

Requirement traceability

A Vision and scope document was produced by Microsoft at the beginning of this project. Below is the traceability from those Vision and scope requirements to the functional requirements. Additional to the Vision and Scope document, detailed functional requirements have been derived, primarily related to configuration decisions, to allow the QA team to construct tests to demonstrate correct configuration of the product for Ernst and Young. This document can be found here: http://sp.ey.net/msibm/MC%20Program/Program%20Management/Envisioning%20Scope%20and%20Vision/Vision%20Scope%20for%20Lync%20-%20Final.docx

Phase 1 Vision and Scope requirements

Business Requirements

| Vision and Scope Requirement Name | Traced Functional or Non-Functional Req ID | Traced Functional or Non-Functional Req name |
| --- | --- | --- |
| LYBR-NF-AP-001 Automatic enabling of AD for Lync | LY-NF-AP-001 LY-NF-AP-002 | New Joiner profile Access Lync IM client without need for any additional provisioning |
| LYBR-NF-AVAIL-001 99.9% availability | LY-NF-AVAIL-002 LY-NF-AVAIL-005 LY-NF-AVAIL-006 LY-NF-AVAIL-012 LY-NF-AVAIL-013 LY-NF-AVAIL-014 | 24/7 global access 99.9% availability of core Lync 99.99% availability of outbound dial Minimum 50,000 Lync users in AMER region 9% per annum growth Minimum of 90,000 Lync users in EMEIA region with 9% per annum growth Minimum of 40,000 Lync users in ASIAPAC region with 9% per annum growth |
| LYBR-NF-AVAIL-003 Consistent performance, scalability and high availability | LY-NF-AVAIL-002 LY-NF-AVAIL-004 LY-NF-AVAIL-005 LY-NF-AVAIL-006 LY-NF-AVAIL-012 LY-NF-AVAIL-013 LY-NF-AVAIL-014 | 24/7 global access Support for minimum 176,000 Lync users 99.9% availability of core Lync 99.99% availability of outbound dial Minimum 50,000 Lync users in AMER region 9% per annum growth Minimum of 90,000 Lync users in EMEIA region with 9% per annum growth Minimum of 40,000 Lync users in ASIAPAC region with 9% per annum growth |
| LYBR-NF-AVAIL-002 AV conferencing and Voice calls restricted if Network bandwidth is limited | LY-NF-AVAIL-007 | AV conferencing and Voice calls restricted if Network bandwidth is limited |
| LYBR-SEC-001 Adherence to EY's Security Policies | LY-SEC-001 LY-SEC-002 LY-SEC-PARM-001 LY-SEC-PARM-002 LY-SEC-PARM-003 LY-SEC-003 LY-SEC-004 LY-SEC-005 LY-SEC-006 LY-SEC-007 | All IM traffic must be encrypted Single sign on for Lync IM Black list for file transfer Disable software tracing P2P App Sharing Encryption Collaboration traffic encrypted using TLS Initial Internal Kerberos authentication will be mandatory All Lync servers shall run anti-virus software Anti-virus scanning of file attachments Use Anti-virus software to inspect all traffic to and from EDGE servers |
| LYBR-VT-LL-002 Retain existing phone number | LY-VT-OD-002 | Retain existing external phone number |
| LYBR-VT-PBX-004 End users to use physical devices to make and receive calls | LY-VT-PBX-001 | Physical phones to make phone calls |

Operational Requirements

| Vision and Scope Requirement Name | Traced Functional or Non-Functional Req ID | Traced Functional or Non-Functional Req name |
| --- | --- | --- |
| LYBR-NF-AVAIL-004 OR01 Component fault tolerance for drives, network adapters, WAN links, routers, switches and hardware load balancers | LY-NF-AVAIL-008 | Component fault tolerance for drives, network adapters, WAN links, routers, switches and hardware load balancers |
| LYBR-NF-INST-001 OR06 Centralized server administration and selective decentralized User Administration | LY-SA-002 | Lync Server 2010 server roles configured based on documented requirements |
| LYBR-IM-INT-001 OR07 Ability to selectively archive Instant Messages of internal users and to always archive communications with Federated Partners over Edge | LY-IM-PARM-017 | Selective retention of chat history for an internal user group |
| LYBR-RA-WEBREM-001 OR08 Ability to selectively enable EY users for external access to Lync functions via Edge (Remote Access) | LY-RA-WEBREM-001 LY-RA-WEBREM-002 | New Joiner profile - EYRC Remote Lync web access to meetings |
| LYBR-NF-OTH-002 OR09 Ability to capture and record Call Detail Records for both Instant Messages and Media Calls | LY-VT-PARM-003 | Enable Call Log Auto Archiving |
| LYBR-NF-AP-002 OR10 Ability to administer the enablement of voice features, including ad-hoc conferencing and call-forwarding, via a phased process for groups and users | LY-SA-001 | Enablement of Voice Features |
| LYBR-NF-INST-002 OR12 Lync-to-Lync file transfers must have a configurable file size limit. | LY-NF-AVAIL-003 | Prohibit the internal transfer via IM of files greater than 15MB |
| LYBR-NF-AVAIL-007 OR14 End user features that have infrastructure impact (such as bandwidth consumption for address book downloads or video calls), must be centrally manageable per user or site. | LY-NF-AVAIL-009 | Central management of address book downloads and video calls |

Security Requirements

| Vision and Scope Requirement Name | Traced Functional or Non-Functional Req ID | Traced Functional or Non-Functional Req name |
| --- | --- | --- |
| LYBR-SEC-003 SEC01 All collaboration traffic will be encrypted using TLS; and internally Kerberos authentication will also be mandatory | LY-SEC-003 LY-SEC-004 | Collaboration traffic encrypted using TLS Initial Internal Kerberos authentication will be mandatory |
| LYBR-NF-OTH-007 SEC02 The use/allowance of archiving, hyperlinks, file transfers, p2p voice and video functionality must be manageable via system administration | No traced requirements | Core design of the product - determined by policies. Will be provided. |
| LYBR-IM-INT-003 SEC03 Archiving feature not enabled for internal conversations | LY-IM-PARM-015 | No retention of chat history for internal IM sessions |
| LYBR-IM-INT-004 SEC04 Conversation History in Outlook is not enabled | LY-IM-PARM-017 LY-IM-PARM-019 | Disable Automatic saving of IM sessions to Outlook Disable the Find Previous Conversations menu |
| LYBR-IM-INT-005 SEC05 Archiving of all federated IM communications | LY-IM-PARM-015 LY-IM-PARM-015 LY-IM-PARM-017 | No retention of chat history for internal IM sessions Selective retention of chat history for an internal user group Disable Automatic saving of IM sessions to Outlook |
| LYBR-SEC-004 SEC07 Ability to control which Lync services are available for each federated partner and authenticated EY users | No traced requirements | Rejected requirement – Lync doesn’t work that way – functionality for one federated partner will be the same for all federated partners |
| LYBR-SEC-006 SEC09 Lync servers must run Anti-Virus and support the ability to scan meeting file attachments or file transfers. | LY-SEC-005 LY-SEC-006 | All Lync servers shall run anti-virus software Anti-virus scanning of file attachments |
| LYBR-NF-INST-003 SEC10 Federation Partners must be manually defined on Edge servers | No traced requirements | Design statement This is the way that Edge Servers work |
| LYBR-SEC-007 SEC13 Compliance to security standards (SASS, logging) | LY-SEC-001 LY-SEC-002 LY-SEC-PARM-001 LY-SEC-PARM-002 LY-SEC-PARM-003 LY-SEC-003 LY-SEC-004 LY-SEC-005 LY-SEC-006 LY-SEC-007 | All IM traffic must be encrypted Single sign in for Lync IM Black list for file transfer Disable software tracing P2P App Sharing Encryption Collaboration traffic encrypted using TLS Initial Internal Kerberos authentication will be mandatory All Lync servers shall run anti-virus software Anti-virus scanning of file attachments Use Anti-virus software to inspect all traffic to and from EDGE servers |
| LYBR-SEC-008 SEC14 Supported ability to inspect all traffic to/from Edge Servers and both Federated and PIC partners | LY-SEC-007 | Use Anti-virus software to inspect all traffic to and from EDGE servers |
| LYBR-SEC-009 SEC16 All messaging clients beyond Outlook must comply with the firm’s security policies | LY-SEC-001 LY-SEC-002 LY-SEC-PARM-001 LY-SEC-PARM-002 LY-SEC-PARM-003 LY-SEC-003 LY-SEC-004 LY-SEC-005 LY-SEC-006 LY-SEC-007 | All IM traffic must be encrypted Single sign in for Lync IM Black list for file transfer Disable software tracing P2P App Sharing Encryption Collaboration traffic encrypted using TLS Initial Internal Kerberos authentication will be mandatory All Lync servers shall run anti-virus software Anti-virus scanning of file attachments Use Anti-virus software to inspect all traffic to and from EDGE servers |
| LYBR-SEC-010 SEC17 The implementation of Lync for Federation must not result in a downloadable or searchable address book accessible to all Federated partners | LY-IM-FED-002 | Prohibit federated correspondents from downloading and/or searching EY address book |

System Requirements

| Vision and Scope Requirement Name | Traced Functional or Non-Functional Req ID | Traced Functional or Non-Functional Req name |
| --- | --- | --- |
| LYBR-NF-SSO-001 SYS01 SIP Domains: will match user SMTP/Email address (for example, user01@cc.ey.com) | LY-NF-OTH-001 | SIP Domains match user SMTP/Email address |
| LYBR-NF-AP-003 SYS02 Support for 145,000 users | LY-NF-AVAIL-004 LY-NF-AVAIL-012 LY-NF-AVAIL-013 LY-NF-AVAIL-014 | Support for minimum 176,000 Lync users Support for minimum 50,000 users in AMER region with 9% per annum growth Support for minimum 90,000 users in EMEIA region with 9% per annum growth Support for minimum 40,000 users in ASIAPAC region with 9% per annum growth |
| LYBR-NF-OTH-004 SYS03 Certificates provided by a CA suitable for use with Microsoft Lync Server 2010 for PKI (http://go.microsoft.com/fwlink/?LinkId=202834) | No traced requirements | Design statement Will be provided in accordance with design |
| LYBR-NF-OTH-005 SYS04 Hardware load balancer that supports Lync Server 2010 requirements for Front End and Director pools (for web traffic only) and Edge Server pool(s) | No traced requirements | Design statement Will be provided in accordance with design |
| LYBR-NF-OTH-006 SYS05 DNS load balancing required for Front End and Director pools (SIP traffic) | No traced requirements | Design statement Will be provided in accordance with design |
| LYBR-SA-003 SYS08 Lync Server 2010 server roles configured based on documented requirements http://technet.microsoft.com/en-us/library/gg398438.aspx | LY-SA-002 | Lync Server 2010 server roles configured based on documented requirements |
| LYBR-NF-AV-001 SYS09 For all Lync Servers running antivirus software, include key Lync Server 2010 directories in the exception list for each security/antivirus application in order to ensure optimal performance | LY-SEC-005 LY-SEC-007 | All Lync servers shall run anti-virus software Use Anti-virus software to inspect all traffic to and from EDGE servers |
| LYBR-NF-AVAIL-009 SYS10 In Front End pool topologies, most servers should be in a network that supports more than 1 Gbps, especially when supporting audio/video (A/V) conferencing and application sharing | No traced requirements | Design statement Will be provided in accordance with design |
| LYBR-NF-AVAIL-010 SYS12 Dedicated Audio Video Conferencing pools in each datacenter | LY-NF-AVAIL-010 | Dedicated Audio Video Conferencing pools in each datacenter |
| LYBR-SEC-002 SYS13 The only Lync Server 2010 server that can be installed on a virtual machine is the Monitoring server | No traced requirements | Design statement Will be provided in accordance with design |
| LYBR-NF-AVAIL-011 SYS14 High availability within a site for IM, presence, conferencing and voice workloads | LY-NF-AVAIL-005 LY-NF-AVAIL-006 | 99.9% availability of core Lync 99.99% availability of outbound dial |
| LYBR-NF-AVAIL-012 SYS21 Two EY Lync users working remotely can exchange secure media peer-to-peer without traversal through EY network | No traced requirements | Design statement Will be provided in accordance with design |
| LYBR-IM-INT-002 SYS25 Supported presentation, searching and adding of the SameTime-equivalent Public Groups from within the Lync client | No traced requirements | Rejected requirement – Lync doesn’t work that way |

Unified Messaging Requirements

| Vision and Scope Requirement Name | Traced Functional or Non-Functional Req ID | Traced Functional or Non-Functional Req name |
| --- | --- | --- |
| LYBR-UM-006 UM01 Unified Messaging must provide call coverage for Lync users in the PBX-replacement Pilot | LY-UM-001 | PBX replacement call coverage |

User requirements

| Vision and Scope Requirement Name | Traced Functional or Non-Functional Req ID | Traced Functional or Non-Functional Req name |
| --- | --- | --- |
| LYBR-CORE-001 UR01 Internal & EYRC user access to Instant Messaging, presence, and conferencing | LY-IM-INT-001 LY-IM-INT-003 LY-IM-PARM-014 LY-WC-INT-001 LY-WC-EXT-003 | Initiate an email from an EY correspondent’s business card Initiate a one-to-one IM chat session with another EY user by clicking on his business card Show presence information from Outlook when Outlook is installed Initiate an internal web conference with other EY IM correspondents External participants able to join web conferences hosted by EY users on Lync |
| LYBR-CORE-002 UR02 Remote EY user access to IM, presence, and conferencing from outside the corporate network via Edge (Remote Access) | LY-RA-WEBREM-001 LY-RA-WEBREM-002 LY-WC-EXT-003 | New Joiner profile - EYRC Remote Lync web access to meetings External participants able to join web conferences hosted by EY users on Lync |
| LYBR-IM-FED-001 UR03 User IM connectivity to directly Federated (via Edge) partners (Microsoft, IBM in Phase 1, others in later phases as approved by legal) | LY-IM-FED-001 | Lync IM for authorized federated partners IBM & Microsoft & AT&T. |
| LYBR-CE-EXCHOUT-002 UR04 Presence and click-to Integration with SharePoint 2010 and Office 2007 and Office 2010 for enabled users | No traced requirements | Requirement belongs to integrated apps – not Lync Expect this to be delivered once integrated apps are delivered to the same environment as Lync. |
| LYBR-CORE-003 UR05 Core Lync services: Desktop Sharing, Lync-to-Lync A/V, Ad-hoc Web and AV Conferencing, File Transfer | LY-IM-INT-009 LY-NF-AVAIL-003 LY-SEC-PARM-001 LY-WC-INT-004 LY-WC-EXT-001 | Enable internal desktop sharing between 2 or more internal EY correspondents Prohibit the internal transfer via IM of files greater than 15MB Black list for file transfer Ability to add audio-conferencing information into a web conferencing invite Integration of audio & video functionality when conferencing with internal correspondents |
| LYBR-VC-FV-001 UR08 Ability to share A/V media with Federated partners | LY-VC-FV-001 | Ability to share A/V media with Federated partners |
| LYBR-CORE-004 UR11 User must be able to click a number displayed in an Office Integrated application or a displayed contact and initiate a voice call | LY-VT-OD-001 LY-VT-OD-004 | Call a correspondent's phone number by clicking on the telephone icon Phone from integrated apps |
| LYBR-VT-LL-001 UR13 Users must have an available local call log to view recent calls (inbound, outbound, missed, etc.) | LY-VT-PARM-003 | Enable Call Log Auto Archiving |
| LYBR-IM-FED-002 UR17 External user access to IM, presence, and conferencing from outside the corporate network via Edge (Remote Access) for all enabled users | LY-WC-EXT-003 LY-RA-WEBREM-001 | External participants able to join web conferences hosted by EY users on Lync New Joiner profile - EYRC |
| LYBR-VT-AUDIO-002 UR30 Ability to add audio-conferencing information (dial-in numbers) into a web conferencing invite | LY-WC-INT-004 | Ability to add audio-conferencing information into a web conferencing invite |
| LYBR-VT-PBX-017 UR34 Supported use of short-dial code for localized global services (Americas Help Desk vs. EMEA Help Desk) | LY-VT-PBX-002 | Short-Dial Codes |
| LYBR-VT-PBX-018 UR35 Supported use of physical devices for Enterprise Voice users | LY-VT-PBX-001 | Physical phones to make phone calls |
| LYBR-VT-PBX-019 UR37 Support of end-user configurable speed-dial buttons on Lync client and supported hard-phones | LY-VT-PBX-003 | Speed dial buttons on Lync client and hard phones |

Phase 1 Requirements to follow

In the interests of allowing QA to start their test planning whilst details of voice policies and transition and operational requirements are determined, the following Phase 1 requirements traceability will be provided in a later version uplift to this document.

Vision and Scope Requirement Name

LYBR-CE-EXCHOUT-001 Business requirement - No Adverse impact on AD and Email

LYBR-VC-001 Business requirement - Record audio and voice conferences

LYBR-VT-PBX-002 Business requirement - Compliance with in-country voice legislation

LYBR-VT-PBX-005 Business requirement - US E911 compliance

LYBR-VT-PBX-006 Business requirement - Voice resiliency where critical to business

LYBR-VT-PBX-007 Business requirement - Retain office short-codes

LYBR-VT-PBX-009 Business requirement - US E911 disclaimer

LYBR-VT-PBX-010 Business requirement - Dial plan permissions

LYBR-VT-001 Business requirement - Monitor and report on quality of voice calls

LYBR-NF-BCK-001 OR02 Clearly defined plans for backup, restore, and disaster recovery scenarios

LYBR-NF-OTH-001 OR03 Clearly defined process for change control

LYBR-NF-AVAIL-005 OR04 Supported use of tools for application aware, predictive monitoring of key metrics and services, as well as diagnostic tools

LYBR-VT-OD-005 SEC06 Prevent Tail End Hop Off (TEHO) where local laws prohibit such calls (routing long distance calls over EY VoIP network to a local PSTN)

LYBR-SEC-005 SEC08 Presented Caller ID and DID for outbound calls must be controlled by administrators and not editable by end-users.

LYBR-NF-INST-003 SEC10 Federation Partners must be manually defined on Edge servers

LYBR-SA-001 SEC11 Support various administrator roles providing only those rights necessary for the role

LYBR-SA-002 SEC12 Central administration of Lync client and server

LYBR-SEC-011 SEC18 Ability to limit services for remote users based upon device, such as EY or non-EY device.

LYBR-VT-OD-003 SYS06 Voice integration with Cisco CallManager 8.x and Alcatel via either Direct SIP or media gateway

LYBR-NF-AVAIL-008 SYS07 The ability to enable QoS on all internal routers supporting Lync Server 2010 AV conferencing or voice traffic.

LYBR-VT-PBX-021 SYS16 Lync voice platform must support integration with analog and analog/digital media gateways, the configuration dialplans, inbound/outbound normalization and policies to govern user enablement

LYBR-NF-AVAIL-013 SYS22 End-user video experience should be manageable via CAC, via user policy and/or device.

LYBR-VT-OD-004 SYS24 Where Tail End Hop Off (TEHO) is enabled, media for Outbound calls must route to the PSTN via the most cost-effective break-out points (such as locally for remote offices) where policy and in-country governance allows

LYBR-VC-002 UR06 Video integration pilot with Tandberg solution such that the existing Tandberg solution and new Lync solution can be connected together to support video and voice.

LYBR-VT-OD-001 UR10 Enterprise voice available to current Cisco/Alcatel IPT users (approx. 90,000 users) for Outbound to PSTN only

LYBR-VT-OD-002 UR12 Client Matter Codes / Forced Authorization codes must be supported for specified PSTN-bound calls

LYBR-VT-PBX-019 UR36 Lync users who have both physical devices and Lync clients should experience feature parity and the same policy governance

Phase 2

The following requirements relate to Phase 2 release of Lync and will be completed in a later uplift to this document

Business Requirements

| Vision and Scope Requirement Name | Traced Functional or Non-Functional Req ID | Traced Functional or Non-Functional Req name |
| --- | --- | --- |
| LYBR-UM-001 Enterprise wide broadcast voicemail messages | To Follow |  |
| LYBR-UM-002 UM integration with IPT | To Follow |  |
| LYBR-UM-003 Exchange UM must be implemented without impact to current usage of GLMS | To Follow |  |
| LYBR-VT-PBX-003 Deployment and use of Common Area Phones | To Follow |  |
| LYBR-VT-PBX-008 Support for paging Systems | To Follow |  |

Operational Requirements

| Vision and Scope Requirement Name | Traced Functional or Non-Functional Req ID | Traced Functional or Non-Functional Req name |
| --- | --- | --- |
| LYBR-NF-AVAIL-006 OR05 AV conferencing and Enterprise voice quality assurance using Monitoring server reporting services | To Follow |  |
| LYBR-NF-OTH-003 OR13 User enablement for Outbound dialing assumes a portal and process is established for users and managers to order headsets and/or other telephony devices | No traced requirements | Not a requirement - purchase will be via standard EY processes Rejected |

System Requirements

| Vision and Scope Requirement Name | Traced Functional or Non-Functional Req ID | Traced Functional or Non-Functional Req name |
| --- | --- | --- |
| LYBR-VT-PBX-022 SYS17 Supported use of “soft DIDs” for numbers assigned to “groups” that do not necessarily ring a particular user but perhaps a Hunt Group | To Follow |  |
| LYBR-VT-PBX-023 SYS18 The voice platform must support multiple points of presence (MPOP) where a user can be logged into the Lync client and receive calls both their and on their supported hard-phone. | To Follow |  |
| LYBR-VT-PBX-024 SYS19 Lync Server must support basic traditional PBX phone services, such as music-on-hold, Boss/Admin delegation, forward all calls, presentation of Caller ID, ability to transfer calls and use of Common Area Phones. | To Follow |  |
| LYBR-VT-PBX-025 SYS20 Lync Server must support enhanced voice features such as Team Ring, Response Groups, Call Park, enhanced Admin Assistant experience, and Emergency Services. | To Follow |  |
| LYBR-VC-LL-002 SYS23 Video standards and experience should be maintained through all phases of the project: | To Follow |  |
| LYBR-VT-PBX-026 SYS26 Supported use of analog phone devices and fax machines with Lync Server platform. | To Follow |  |

Unified Messaging Requirements

| Vision and Scope Requirement Name | Traced Functional or Non-Functional Req ID | Traced Functional or Non-Functional Req name |
| --- | --- | --- |
| LYBR-UM-006 UM01 Unified Messaging must provide call coverage for Lync users in the PBX-replacement Pilot | LY-UM-001 | PBX replacement call coverage |
| LYBR-UM-007 UM02 Unified Messaging must provide Message Waiting Indicator services | LY-UM-002 | Message Waiting Indicator |
| LYBR-UM-008 UM03 Unified Messaging must provide Telephony User Interface services for accessing voicemail via a physical phone | LY-UM-003 | Voicemail via a physical phone |
| LYBR-UM-009 UM04 Unified Messaging must provide a voice recognition service for searching and calling a user | To Follow |  |
| LYBR-UM-010 UM05 Exchange Unified Messaging must support administration via policy for message retention | To Follow |  |
| LYBR-UM-011 UM06 Exchange Unified Messaging voicemail messages should not be transcribed into emails | To Follow |  |
| LYBR-UM-012 UM07 Exchange Unified Messaging voicemails must not be archived | To Follow |  |
| LYBR-UM-013 UM08 Exchange Unified Messaging must support the display of Caller ID and must resolve to a user name for internal callers | LY-UM-004 LY-UM-005 | Caller ID display Caller id resolve to user name |
| LYBR-UM-014 UM09 Exchange Unified Messaging must be configured to prevent voicemails from being forwarded outside of the organization | No traced requirements | Q&RM is allowing for the forward of VM messages Rejected |
| LYBR-UM-015 UM10 Exchange Unified Messaging must be configured to prevent voicemails from being forwarded within the organization by both group and voicemail type | To Follow |  |
| LYBR-UM-016 UM11 Exchange Unified Messaging must be configured to prevent the local download of voicemail media files by users | To Follow |  |
| LYBR-UM-017 UM12 Ability for preservation of voicemail during legal hold | LY-UM-006 | Ability for preservation of voicemail during legal hold |
| LYBR-UM-018 UM13 Must have the ability to disable voice mailbox access and de-provision a user without deleting the voice mailbox. | To Follow |  |
| LYBR-UM-019 UM14 Need to ability to allow someone else to access a voice mailbox in the event of an investigation or legal requirement. | To Follow |  |

User Requirements

| Vision and Scope Requirement Name | Traced Functional or Non-Functional Req ID | Traced Functional or Non-Functional Req name |
| --- | --- | --- |
| LYBR-VC-LL-001 UR07 EY users must be able to browse a video endpoint directory (EY Video Directory) to initiate video calls to EY video endpoints | LY-VC-LL-001 | Video endpoint directory |
| LYBR-VT-PBX-011 UR09 Enterprise Voice (PBX replacement) as Proof-of-Concept only for certain users/locations | To Follow |  |
| LYBR-VT-AUDIO-004 UR16 Piloted use of Dial-in Audio Conferencing | To Follow |  |
| LYBR-WC-EXT-001 UR18 External participants able to join web conferences hosted by EY users on Lync | LY-WC-EXT-003 | External participants able to join web conferences hosted by EY users on Lync |
| LYBR-WC-EXT-002 UR19 Full Availability of A/V when conferencing with Federated partners to all users | LY-WC-EXT-004 | Full Availability of A/V when conferencing with Federated partners to all users |
| LYBR-VC-LV-001 UR20 Completed integration with Tandberg video conferencing equipment, services available to all users | To Follow |  |
| LYBR-VT-PBX-012 UR22 Full Enterprise Voice (PBX replacement) | To Follow |  |
| LYBR-VT-PBX-013 UR23 Ability to leverage Boss/Admin features during PBX replacement, such as ability to transfer a call directly to voicemail, call on-behalf of and shared-line appearance. | To Follow |  |
| LYBR-VT-PBX-014 UR24 Ability for receptionists to leverage an enhanced Lync client experience (Attendant Console) for managing multiple lines and consistent consultative transfers, etc. | To Follow |  |
| LYBR-UM-004 UR25 Unified Messaging for current Lyrix users (approx. 50,000 users), providing call coverage to users on Cisco IPT (migration of voicemails is out-of-scope) | To Follow |  |
| LYBR-IM-FED-003 UR27 Integration with public instant messaging providers – AOL, Yahoo and MSN as approved by legal | LY-IM-FED-003 | FR 3.5.1 Integration with public IM providers - AOL, Yahoo and MSN |
| LYBR-VT-AUDIO-001 UR29 Production deployment of Dial-in Audio Conferencing, possibly with Hosted integration (TBD) | To Follow |  |
| LYBR-VT-PBX-027 UR32 Full Enterprise Voice (PBX replacement) for all users/locations | To Follow |  |
| LYBR-VT-PBX-016 UR33 Supported use of “Hot Desking” where users can login either manually or automatically via a physical connection to a deskphone in a “hotel” cube | To Follow |  |
| LYBR-UM-005 UR38 Unified Messaging for all users | To Follow |  |

Phase 3 – all requirements to follow

The following requirements relate to Phase 3 release of Lync and will be provided in a later uplift to this document

Vision and Scope Requirement Name

LYBR-NF-DR-001 Contact centre services potential replacement

LYBR-VT-PBX-001 Auto Attendant Services

LYBR-MOB-LM-001 UR14 Supported use of the Lync Mobile client in a Pilot (upon release)

LYBR-IM-PCHAT-001 UR15 Group Chat as a Pilot only

LYBR-MOB-LM-002 UR21 Supported use of the Lync Mobile client for Instant Messaging for all mobile users

LYBR-IM-PCHAT-002 UR26 Group Chat for production deployment (possibly Wave 15 unified client, TBD)

LYBR-VT-PBX-015 UR28 Contact Center services should either be replaced with comparable service or maintained without interruption at the introduction of Lync Server

LYBR-VT-AUDIO-003 UR31 Supported ability for conference organizers to leverage user and chair PINs.

LYBR-WC-EXT-003 OR11 Ability to manage and administer the “Dial-Back” feature of Dial-in Audio Conferencing for geographic/cost considerations

Requirement descriptions

Co-Existence

| Req ID | Req Parent | Name | Description | Phase |
| --- | --- | --- | --- | --- |
| 748 | CE - Exchange - Outlook | LY-CE-PARM-001 Sync Lync contacts with Exchange | The system shall create a corresponding personal contact in Microsoft Outlook for each person on a user’s Lync 2010 Contacts list Requires Outlook Parameter=EnableExchangeContactSync Setting=Null | Phase 1 |
| 749 | CE - Other | LY-CE-PARM-002 Enable One Note 12 Integration | The option to Take Notes Using One Note shall be enabled Note - not part of standard EY tool set so will only work if One Note is installed GCL5 includes, hidden in GCL4 Requires Office Parameter=DisableOneNote12Integration Setting=Null | Phase 1 |

Instant Messaging

| Req ID | Req Parent | Name | Description | Phase |
| --- | --- | --- | --- | --- |
| 750 | IM - Federated | LY-IM-FED-001 Lync IM for authorized federated partners IBM & Microsoft & AT&T. | Federated partners IBM, Microsoft and AT&T shall be permitted external access to Lync Requires Federated party | Pilot |
| 751 | IM - Federated | LY-IM-FED-002 Prohibit federated correspondents from downloading and/or searching EY address book | The system shall prohibit federated correspondents from downloading and/or searching EY address book Inherent in architecture - would see presence | Pilot |
| 752 | IM - Federated | LY-IM-FED-003 Integration with public IM providers - AOL, Yahoo and MSN | The system shall provide Integration with public IM providers - AOL, Yahoo and MSN Test on GCL 4 and 5 | Phase 3 |
| 762 | IM - Federated | LY-IM-PARM-001 Display Federated user SIP address | The system shall display a federated contact's SIP address when notifying a user that the federated contact has added them to their contact list Parameter=DisableFederatedPromptDisplayName Setting=TRUE | Pilot |
| 763 | IM - Federated | LY-IM-PARM-002 Display PIC user SIP address | The system shall display a PIC contact's SIP address when notifying a user that the PIC contact has added them to their contact list Parameter=DisablePICPromptDisplayName Setting=TRUE | Phase 3 |
| 753 | IM - Internal | LY-IM-INT-001 Initiate an email from an EY correspondent’s business card | The user shall be able to Initiate an email from an EY correspondent’s business card Requires Email client | Pilot |
| 754 | IM - Internal | LY-IM-INT-002 The business card shall display:Names | The business card shall display: Correspondent’s name as sourced from the Active Directory Requires AD Not middle initial unless makes unique | Pilot |
| 755 | IM - Internal | LY-IM-INT-003 Initiate a one-to-one IM chat session with another EY user by clicking on his business card | The user shall be able to initiate a one-to-one IM chat session with another EY user by clicking on his business card | Pilot |
| 756 | IM - Internal | LY-IM-INT-004 Correspondent’s availability status warning | The user shall be warned of the correspondent’s availability status, if not available for a chat session, before giving them the option to initiate a chat session anyway | Pilot |
| 757 | IM - Internal | LY-IM-INT-005 LY-IM-INT-005 Core Instant Messaging n-way Chat with other EY users by dragging into an existing chat session | The user shall be able to instigate Core Instant Messaging n-way Chat with other EY users by dragging the users’ name into an existing chat session | Pilot |
| 758 | IM - Internal | LY-IM-INT-006 Business card shall display: locations | The business card shall display: Correspondent’s geographical locations as sourced from the Active Directory on hovering over an EY User’s name on the IM contact screen Requires AD Is no secondary location? | Pilot |
| 759 | IM - Internal | LY-IM-INT-007 Business card shall display: of email and telephone | The business card shall display: Correspondent’s email and telephone details as sourced from the Active Directory on hovering over an EY User’s name on the IM contact screen Requires AD | Pilot |
| 760 | IM - Internal | LY-IM-INT-008 Business card shall display: IM Webpage photograph | The business card shall display: IM Webpage photograph (if available) on hovering over an EY User’s name on the IM contact screen Requires AD | Pilot |
| 761 | IM - Internal | LY-IM-INT-009 Enable internal desktop sharing between 2 or more internal EY correspondents | The System shall enable internal desktop sharing between 2 or more internal EY correspondents | Pilot |
| 764 | IM - Internal | LY-IM-PARM-003 Meeting subject or location Option | The user shall be able to see another user's meeting subject or location if the user has enabled the option Test on GCL 4 and 5 Requires Outlook Parameter=DisableMeetingSubjectAndLocation Setting=Null | Pilot |
| 765 | IM - Internal | LY-IM-PARM-004 Rich text formatting in an instant message | The user shall be able to use rich text formatting in an instant message Parameter=DisableRTFIM Setting=Null | Pilot |
| 766 | IM - Internal | LY-IM-PARM-005 Disable enterprise customised help menu links | The user shall not be able to access enterprise customised help menu links Parameter=CustomizedHelpUrl Setting=Null | Pilot |
| 767 | IM - Internal | LY-IM-PARM-006 Notification of adding to contact list disabled | The user shall not be notified when someone adds them to their contact list Parameter=EnableNotificationForNewSubscribers Setting=FALSE | Pilot |
| 768 | IM - Internal | LY-IM-PARM-007 Access a website or location via URL in IM | The user shall be able to enter a URL to an instant message and it shall be made clickable Parameter=EnableUrl Setting=Null | Pilot |
| 769 | IM - Internal | LY-IM-PARM-008 Search outlook folders for contacts | The user shall be able to exclude specific outlook folders from the search outlook folders for contacts Can do the search Parameter=ExcludedContactFolders Setting=Null | Pilot |
| 770 | IM - Internal | LY-IM-PARM-009 Outlook and Lync sync | The system shall ensure that if different users are logged into a single machine for outlook and Lync that Lync only picks up the data of the user logged into Lync Requires Outlook Authentication should stop this being possible Parameter=DisableEmailComparisonCheck Setting=Null | Pilot |
| 771 | IM - Internal | LY-IM-PARM-010 Send and receive emoticons | The user shall be able to send and receive standard emoticons Parameter=DisableEmoticons Setting=Null | Pilot |
| 772 | IM - Internal | LY-IM-PARM-011 No custom presence state other than the standard Available, Busy and Do not Disturb and Appear Offline | The user shall not be able to set a presence state other than the standard Available, Busy and Do not Disturb and Appear Offline Parameter=CustomStateUrl Setting=Null | Pilot |
| 773 | IM - Internal | LY-IM-PARM-012 Presence state of Appear Offline | The user shall be able to set the presence state of Appear Offline Parameter=EnableAppearOffline Setting=Null | Pilot |
| 774 | IM - Internal | LY-IM-PARM-013 No calendar presence when Outlook is not installed | The system shall not attempt to get calendar presence when Outlook is not installed Parameter=DisableCalendarPresence Setting=Null | Pilot |
| 775 | IM - Internal | LY-IM-PARM-014 Show presence information from Outlook when Outlook is installed | The system shall show presence information from Outlook when Outlook is installed Parameter=DisableCalendarPresence Setting=Null | Pilot |
| 776 | IM - Internal | LY-IM-PARM-015 No retention of chat history for internal IM sessions | The system shall not allow the retention of chat history for internal IM sessions - Note there is a very small group of users for whom the server-side will need to retain messages. There is a separate requirement for this Parameter=DisableSavingIM Setting=TRUE | Pilot |
| 777 | IM - Internal | LY-IM-PARM-016 Selective retention of chat history for an internal user group | the system shall permit selective server side retention of chat history for an internal user group Parameter=CsArchivingPolicy & CsArchivingConfiguration Setting= | Pilot |
| 778 | IM - Internal | LY-IM-PARM-017 Disable Automatic saving of IM sessions to Outlook | the system shall Disable Automatic saving of IM sessions to Outlook Requires Outlook Parameter=EnableIMAutoArchiving Setting=FALSE | Pilot |
| 779 | IM - Internal | LY-IM-PARM-018 No legal/Disclaimer to any internal IM conversations | The system shall not Add a legal/Disclaimer to any internal IM conversations Parameter=IMWarning Setting=Null | Pilot |
| 780 | IM - Internal | LY-IM-PARM-019 Disable the Find Previous Conversations menu | The system shall disable the Find Previous Conversations menu Parameter=DisableOnlineContextualSearch Setting=Null | Pilot |
| 781 | IM - Internal | LY-IM-PARM-020 Disable Event logging | Event logging shall be disabled as a default Parameter=EnableEventLogging Setting=Null | Pilot |
| 782 | IM - Internal | LY-IM-PARM-021 User override the windows event logging parameter | The user shall be able to override the windows event logging parameter and turn it on Parameter=EnableEventLogging Setting=Null | Pilot |
| 783 | IM - Internal | LY-IM-PARM-022 Disable the contact card organisation tab | The system shall disable the contact card organisation tab Parameter=DisableContactCardOrganizationTab Setting=TRUE | Pilot |
| 784 | IM - Internal | LY-IM-PARM-023 The user shall be able to turn display of free/busy info on or off | The user shall be able to turn display of free/busy info on or off Test on GCL 4 and 5 Requires Outlook Parameter=DisableFreeBusyInfo Setting=Null | Pilot |
| 785 | IM - Internal | LY-IM-PARM-024 Show availability based on Outlook calendar | If Free/busy parameter is turned on the system shall show availability based on their Outlook calendar Test on GCL 4 and 5 Requires Outlook Parameter=DisableCalendarPresence Setting=Null | Pilot |
| 786 | IM - Internal | LY-IM-PARM-025 Show the user as busy whenever they are using their Lync phone | The system shall show the user as busy whenever they are using their Lync phone Parameter=DisablePhonePresence Setting=Null | Pilot |
| 787 | IM - Internal | LY-IM-PARM-026 User can turn display of out of office message from Outlook on or off | When Outlook is installed the user shall be able to turn display of out of office message from Outlook on or off Test on GCL 4 and 5 Requires Outlook Parameter=DisablePresenceNote Setting=Null | Pilot |
| 788 | IM - Internal | LY-IM-PARM-027 Display the Out of office message from outlook on hover over the business card | If presence Note (Out Of Office indicator) is enabled the system shall display the Out of office message from outlook on hover over the business card Test on GCL 4 and 5 Requires Outlook Parameter=DisablePresenceNote Setting=Null | Pilot |
| 789 | IM - Internal | LY-IM-PARM-028 Display the user's photo sourced from AD as a default | The system shall display the user's photo sourced from AD as a default Test on GCL 4 and 5 Requires AD & Sharepoint Parameter=DisplayPhoto Setting=PhotosFromADOnly | Pilot |
| 790 | IM - Internal | LY-IM-PARM-029 Display the photos of a user's contacts sourced from AD | The system shall display the photos of a user's contacts sourced from AD Test on GCL 4 and 5 Requires AD & Sharepoint Parameter=DisplayPhoto Setting=PhotosFromADOnly | Pilot |
| 791 | IM - Internal | LY-IM-PARM-030 Allow a user to change their photo by browsing to a replacement location | The system shall allow a user to change their photo by browsing to a replacement location Test on GCL 4 and 5 Requires AD & Sharepoint Parameter=DisplayPhoto Setting=PhotosFromADOnly | Pilot |
| 792 | IM - Internal | LY-IM-PARM-031 Maximum of 150 contacts | The user shall be able to add a maximum of 150 contacts Need to discuss further Need to discuss further Parameter=MaximumNumberOfContacts Setting=150 | Pilot |
| 793 | IM - Internal | LY-IM-PARM-032 Maximum of 10 distribution lists | The user shall be able to create a maximum of 10 distribution lists Need to discuss further Need to discuss further Parameter=MaximumDGsAllowedInContactList Setting=10 | Pilot |
| 794 | IM - Internal | LY-IM-PARM-033 Photo of maximum size 30KB | The user shall be able to add a photo of maximum size 30KB Sharepoint requirement Parameter=MaxPhotoSizeKb Setting=30 | Pilot |
| 795 | IM - Internal | LY-IM-PARM-034 See and amend relationships status for contacts | The user shall be able to see and amend relationships status for their contacts Not sure if true or false Parameter=ShowManagePrivacyRelationships Setting=Null | Pilot |
| 871 | IM - Internal | LY-IM-PARM-035 Edit photo via a link to user SharePoint site | When SharePoint is available the user shall be able to edit their photo via a link to their SharePoint site Requires Sharepoint Parameter=ShowSharepointPhotoEditLink Setting=TRUE | Pilot |

Remote Access

| Req ID | Req Parent | Name | Description | Phase |
| --- | --- | --- | --- | --- |
| 797 | RA - Web Access Remote | LY-RA-WEBREM-001 New Joiner profile - EYRC | New Joiner profile shall include Lync IM access via EYRC Provisioning Requirement | Phase 1 |
| 798 | RA - Web Access Remote | LY-RA-WEBREM-002 Remote Lync web access to meetings | Users without Lync shall be able to join meetings using the Lync Web App | Phase 2 |

Security

| Req ID | Req Parent | Name | Description | Phase |
| --- | --- | --- | --- | --- |
| 799 | Security | LY-SEC-001 All IM traffic must be encrypted | The system shall encrypt all IM traffic | Pilot |
| 800 | Security | LY-SEC-002 Single sign On for Lync IM | EY employees must be able to use a single identity to access Lync IM Note the sign in name is different for Lync than windows, so they will have to type this the first time. However, the account is the same, so they don't need a password | Pilot |
| 866 | Security | LY-SEC-003 Collaboration traffic encrypted using TLS | the system shall encrypt Collaboration traffic using TLS | Pilot |
| 867 | Security | LY-SEC-004 Initial Internal Kerberos authentication will be mandatory | The system shall make initial internal authentication via Kerberos mandatory. | Pilot |
| 868 | Security | LY-SEC-005 All Lync servers shall run anti-virus software | The sysrtem shall run antivirus software on all servers | Pilot |
| 869 | Security | LY-SEC-006 Anti-virus scanning of file attachments | All file attachments shall be scanned by the anti-virus software | Pilot |
| 870 | Security | LY-SEC-007 Use Anti-virus software to inspect all traffic to and from EDGE servers | the system shall anti-virus inspect all traffic to/from Edge Servers | Phase 1 |
| 970 | Security | LY-SEC-008 Sec Incident - telephony databases - notice | EY shall be able to access the SQL Telephony Databases –(CDR and QOE) at 1 hours notice | Pilot |
| 971 | Security | LY-SEC-009 Sec Incident - telephony databases - data age | EY shall be able to access data from the SQL Telephony Databases up to 52 weeks old | Pilot |
| 972 | Security | LY-SEC-010 Sec Incident - telephony databases - Information to Access | EY shall be able to access the following information on the SQL Telephony Databases: Who accessed the system, what they accessed, who was part of the communication, duration , phone numbers, email addresses, time of activity, where from, where to, IP addresses, SIP URI | Pilot |
| 973 | Security | LY-SEC-011 Sec Incident - telephony databases - Search Criteria | EY shall be able to search for relevant log entries based on : User name, user email, range of time of events, participants in the communication, type of communication (IM, Voice, web conf, video conf), phone number calling from, phone number calling to, SIP URI, IP address | Pilot |
| 974 | Security | LY-SEC-012 Sec Incident - telephony databases - Data format | The information from the logs shall be available in readable and searchable format | Pilot |
| 975 | Security | LY-SEC-013 Sec Forensic Investigation - Notice | EY shall be able to have access to data to support a forensic investigation at immediate notice (regulatory, governmental requirements) | Pilot |
| 976 | Security | LY-SEC-014 Sec Forensic Investigation - List of Log files | EY shall have access to server extracts of the following log files SECURITY LOGS: Windows Security Event Log Windows CAPI2 (CryptoAPI 2.0) Event Log for PKI issues IIS access logs FIREWALL Export of these logs will be real time – export via Syslog UDP port 514 SYSTEM ACTIVITY LOGS SQL Telephony Databases –CDR and QOE (via flat file export) Lync Server event logs | Pilot |
| 977 | Security | LY-SEC-015 Sec Forensic Investigation - Server types and infrastructure | EY shall have access to the server extracts for the following server types and infrastructure Lync Servers: Edge, Front End, Director, A/V, File Share, Administration, SQL, Mediation, Monitoring, Archiving, Active Directory, SCOM Gateway, SCOM Lync Monitors Infrastructure: Firewalls | Pilot |
| 978 | Security | LY-SEC-016 Sec Forensic Investigation - Data format | The server extracts shall be provided in raw format (unchanged from how it's stored) | Pilot |
| 979 | Security | LY-SEC-017 Sec Forensic Investigation - Search Criteria | EY shall have access to relevant logs based on Range of time of events, relevant datacentre | Pilot |
| 980 | Security | LY-SEC-018 Sec Forensic Investigation - Data Age | EY shall be able to access the log files up to 52 weeks old | Pilot |
| 801 | Security | LY-SEC-PARM-001 Black list for internal file transfer | The system shall prohibit the internal transfer via IM of proscribed files Can we differentiate internal and federated Parameter=CsFileTransferFilterConfiguration Setting=TBA | Pilot |
| 802 | Security | LY-SEC-PARM-002 Disable software tracing | Software tracing shall not be enabled as a default This needs an operational process to ensure proper controls are in place if it is ever requested Parameter=EnableTracing Setting=Null | Pilot |
| 803 | Security | LY-SEC-PARM-003 P2P App Sharing Encryption | desktop and application sharing data exchanged during a peer-to-peer conversation shall be encrypted where possible Parameter=P2PAppSharingEncryption Setting=Supported | Pilot |

System Administration

| Req ID | Req Parent | Name | Description | Phase |
| --- | --- | --- | --- | --- |
| 864 | System Administration | LY-SA-001 Enablement of Voice Features | The system administrators shall be able to administer the enablement of voice features, including ad-hoc conferencing and call-forwarding, via a phased process for groups and users | Pilot |
| 865 | System Administration | LY-SA-002 Lync Server 2010 server roles configured based on documented requirements | Lync Server 2010 server roles shall be configured based on documented requirements http://technet.microsoft.com/en-us/library/gg398438.aspx | Phase 1 |

Unified Messaging

| Req ID | Req Parent | Name | Description | Phase |
| --- | --- | --- | --- | --- |
| 804 | Unified Messaging | LY-UM-001 Call coverage | The system shall provide a way of handling missed calls by routing to voicemail or other existing call coverage paths Note; In Phase One, in locations where inbound dial is being provided call coverage should be handled by the existing telephony system. In subsequent phases, Exchange UM will provide voice mail as part of Lyrix voicemail replacement and as part of PBX replacement. | Phase 1;Phase 2 |
| 805 | Unified Messaging | LY-UM-002 Message Waiting Indicator | Unified Messaging shall provide Message Waiting Indicator services Requires Unified Messaging | Phase 2 |
| 806 | Unified Messaging | LY-UM-003 Voicemail via a physical phone | Unified Messaging shall provide Telephony User Interface services for accessing voicemail via a physical phone Requires Unified Messaging | Phase 2 |
| 807 | Unified Messaging | LY-UM-004 Caller ID display | Unified Messaging must support the display of Caller ID Requires Unified Messaging | Phase 2 |
| 808 | Unified Messaging | LY-UM-005 Caller id resolve to user name | Caller ID shall resolve to a user name Requires Unified Messaging | Phase 2 |
| 809 | Unified Messaging | LY-UM-006 Ability for preservation of voicemail during legal hold | the system shall be able to preserve voicemail during legal hold proceedings Test on GCL 4 and 5 Requires Unified Messaging | Phase 2 |

Video Conferencing

| Req ID | Req Parent | Name | Description | Phase |
| --- | --- | --- | --- | --- |
| 810 | VC - Federated Video | LY-VC-FV-001 Ability to share A/V media with Federated partners | The user shall be able to share audio/visual media with federated partners | Phase 2 |
| 811 | VC - Lync to Lync | LY-VC-LL-001 Video endpoint directory | EY Users shall be able to browse a video endpoint directory (EY Video Directory) to initiate video calls to EY video endpoints Note this is a phase 2 requirement but it probably won’t be possible. Possible solution is to load video end points into AD. Review in Phase 2 | Phase 2 |
| 812 | VC - Lync to Lync | LY-VC-LL-002 Initiate a video call to the correspondent | The EY user shall be able to initiate a video call to the correspondent by clicking on the video call icon by hovering over an EY correspondent’s business card, Test note, ensure the correspondent can receive the video call as part of the test scenario | Pilot |
| 983 | VC - Lync to Lync | LY-VC-LL-003 Video - share a screen while making a video call | The user shall be able to share a screen whilst on a video call | Pilot |
| 815 | VC - Lync to Lync | LY-VC-PARM-001 Provide warnings if a video device is not working correctly | the system shall provide warnings if a video device is not working correctly Requires Video device Parameter=DisablePoorDeviceWarnings Setting=Null | Pilot |
| 816 | VC - Lync to Lync | LY-VC-PARM-002 Disable Full screen Video | The system shall disable Full screen Video Note that the user will be able to switch to full screen if they want to Parameter=EnableFullScreenVideo Setting=Null | Pilot |
| 817 | VC - Lync to Lync | LY-VC-PARM-003 Enable video in preview mode | The system shall enable video in preview mode Parameter=EnableFullScreenVideo Setting=Null | Pilot |
| 872 | VC - Lync Video with EY | LY-VC-LV-001 Initiate a video call to an entry on the EY video directory | The user shall be able to initiate a video call to an entry on the EY video directory by clicking on the corresponding directory entry. | Pilot |
| 873 | VC - Lync Video with EY | LY-VC-LV-002 Integrate with the Tandberg video conferencing. | The System shall be able to integrate with the Tandberg video conferencing. Requires Tandberg | Phase 2 |
| 982 | VC - Lync Video with EY | LY-VC-LV-003 Video - integrate with cisco VC infrastructure | The system shall integrate with Cisco VC infrastructure - Bridges and gateways | Phase 2 |

Voice Telephony

| Req ID | Req Parent | Name | Description | Phase |
| --- | --- | --- | --- | --- |
| 1014 | VT - Lync to Lync | LY-VT-LL-001 Voice - Voice Policies | The system shall support all possible variants of voice policy as detailed in the Lync Enterprise Voice - Voice Policies Document located in http://sp.ey.net/msibm/MC%20Program/Lync/03.%20Engineering/Enterprise%20Voice/ | Pilot |
| 821 | VT - Lync to Lync | LY-VT-PARM-001 Enable calls across the internet | The user shall be able to make calls across the internet where the network permits Note this is just lync-to-lync calls where bandwidth / network permits Parameter=DisableICE Setting=Null | Pilot |
| 822 | VT - Lync to Lync | LY-VT-PARM-002 Enable Poor Network Warnings | The user shall be able to see a network status indicator Parameter=DisablePoorNetworkWarnings Setting=Null | Pilot |
| 823 | VT - Lync to Lync | LY-VT-PARM-003 Enable Call Log Auto Archiving | The system shall save a log of calls made to the user's outlook folder Test on GCL 4 and 5 Requires Outlook Q&RM decision Parameter=EnableCallLogAutoArchiving Setting=Null | Pilot |
| 824 | VT - Lync to Lync | LY-VT-PARM-004 Disable Client Music On Hold | The user shall not hear music when a call is put on hold Parameter=EnableClientMusicOnHold Setting=Null | Pilot |
| 825 | VT - Lync to Lync | LY-VT-PARM-005 Enable Click to Call Default | The initial setting for click to call shall place a native Lync call by default whenever a user employs the click-to-call feature Requires Enterprise Voice Parameter=EnableVOIPCallDefault Setting=Null | Pilot |
| 826 | VT - Lync to Lync | LY-VT-PARM-006 Music On Hold Audio File | The system shall not specify nor allow the user to specify a music on hold audio file Parameter=MusicOnHoldAudioFile Setting=Null | Pilot |
| 818 | VT - Outbound Dial PSTN | LY-VT-OD-001 Call a correspondent's phone number by clicking on the telephone icon | On hovering over an EY correspondent's business card, the EY user shall be able to call a correspondent's phone by clicking on the telephone icon | Pilot |
| 819 | VT - Outbound Dial PSTN | LY-VT-OD-002 Retain existing external phone number | During the Outbound only voice deployment IPT users shall retain their existing outbound number presentation on Lync Server 2010 | Pilot |
| 820 | VT - Outbound Dial PSTN | LY-VT-OD-003 Enterprise Voice for Outbound dial | The user shall be able to utilise Enterprise voice for Outbound to PSTN calls | Phase 1 |
| 981 | VT - Outbound Dial PSTN | LY-VT-OD-004 Phone from integrated apps | The User with Enterprise voice enabled shall be able to click a number displayed in an Office Integrated application or a displayed contact and initiate a voice call Test on GCL 4 and 5 Requires Office apps | Pilot |
| 827 | VT - PBX Replacement | LY-VT-PBX-001 Physical phones to make phone calls | The user shall be able to use a physical device to make a Lync call i.e. A phone rather than through their computer Note not all users will have physical phones, however need to prove that it does work Requires PBX Replacement | Phase 1 |
| 828 | VT - PBX Replacement | LY-VT-PBX-002 Short-Dial Codes | Supported use of short-dial code for localized global services (Americas Help Desk vs. EMEA Help Desk) Requires Enterprise Voice | Phase 1 |
| 829 | VT - PBX Replacement | LY-VT-PBX-003 Speed dial buttons on Lync client and hard phones | Support of end-user configurable speed-dial buttons on Lync client and supported hard-phones Note Lync doesn't actually support speed dial buttons - will be replaced by contact lists | Phase 1 |
|  | VT - Outbound Dial PSTN | LY-VT-OD-005 Outbound dial - call work number not Lync to Lync | When the User selects the work number in the Lync client, it will dial the desk phone and not dial Lync to Lync. Requires: Enterprise Voice |  |

Web Conferencing

| Req ID | Req Parent | Name | Description | Phase |
| --- | --- | --- | --- | --- |
| 874 | WC - External | LY-WC-EXT-001 Integration of audio & video functionality when conferencing with internal correspondents | The System shall enable the integration of audio & video functionality when conferencing with internal correspondents | Phase 1 |
| 875 | WC - External | LY-WC-EXT-002 Integration of audio & video functionality when conferencing with external federated correspondents | The System shall enable the integration of audio & video functionality when conferencing with external federated correspondents | Phase 2 |
| 876 | WC - External | LY-WC-EXT-003 External participants able to join web conferences hosted by EY users on Lync | Authorised External participants shall be able to join web conferences hosted by EY users on Lync Requires Federation | Phase 2 |
| 877 | WC - External | LY-WC-EXT-004 Full Availability of A/V when conferencing with Federated partners to all users | Full audio/visual shall be available to conferences with federated users Requires Federation | Phase 2 |
| 834 | WC - Internal | LY-WC-INT-001 Initiate an internal web conference with other EY IM correspondents | The system shall enable a user to initiate an internal web conference with other EY IM correspondents by clicking on the appropriate Business card icon. | Pilot |
| 835 | WC - Internal | LY-WC-INT-002 Enable dial out from the web conference | The System shall enable dial out from the web conference Phase 2 | Phase 2 |
| 836 | WC - Internal | LY-WC-INT-003 Enable web based controls for audio (Mute, raise volume, etc) when dialling out from a conference | The System shall enable web based controls for audio (Mute, raise volume, etc) when dialling out from a conference Phase 2 | Phase 2 |
| 837 | WC - Internal | LY-WC-INT-004 Ability to add audio-conferencing information into a web conferencing invite | The user shall be able to add audio-conferencing information into a web conferencing invite Requires Audi Conferencing | Phase 3 |
| 838 | WC - Internal | LY-WC-PARM-001 Provide warnings if an audio device is not working correctly | the system shall provide warnings if an audio device is not working correctly Parameter=DisablePoorDeviceWarnings Setting=Null | Pilot |

Non-Functional

| Req ID | Req Parent | Name | Description | Phase |
| --- | --- | --- | --- | --- |
| 863 | Lync Non-Functional Requirements | LY-NF-PARM-001 HTML permitted in IM | HTML formatting shall be retained when pasted into an IM message Parameter=DisableHtmlIm Setting=Null | Pilot |
| 839 | NF - Account Provisioning | LY-NF-AP-001 New Joiner profile | New Joiner profile shall include Lync IM access Provisioning Requirement | Pilot |
| 840 | NF - Account Provisioning | LY-NF-AP-002 Access Lync IM client without need for any additional provisioning | During the migration period EY employees shall be able to access Lync IM client without need for any additional provisioning | Pilot |
| 841 | NF - Account Provisioning | LY-NF-AP-003 Lync Activation - Automated | The system shall activate dormant Lync Clients without need for user intervention Test on GCL 4 and 5 | Pilot |
| 842 | NF - Account Provisioning | LY-NF-AP-004 Lync activation - User targeted | The system activation shall be able to be targeted by HR attributes out of Active Directoy (to include geography, or sub-area or office) Test on GCL 4 and 5 | Pilot |
| 843 | NF - Account Provisioning | LY-NF-AP-005 Lync Activation - Timing | The system activation shall be able to be targeted at a specific time Test on GCL 4 and 5 | Pilot |
| 844 | NF - Account Provisioning | LY-NF-AP-006 Sametime Deactivation - automatic | The system shall automatically hide sametime from all users Test on GCL 4 and 5 | Pilot |
| 845 | NF - Account Provisioning | LY-NF-AP-007 Sametime Deactivation - doesn't require machine to be shut down to remove it | When removing sametime it shall not be necessary for the user to shut their machine down in order for the change to take place. Test on GCL 4 and 5 | Pilot |
| 846 | NF - Account Provisioning | LY-NF-AP-008 Directory preparation - phone numbers | All mandatory phone numbers in Active directory shall be populated before those users are activated | Pilot |
| 848 | NF - Account Provisioning | LY-NF-AP-010 Directory preparation - contact card | Information on the Lync contact card shall be correctly sourced from Active Directory where populated: Department, Office work number, mobile number, email and location timezone | Pilot |
| 849 | NF - Account Provisioning | LY-NF-AP-011 Lync Activation - contact lists | The user shall be able to access an offline list of their existing Sametime Buddies when they have their Lync client activated Test on GCL 4 and 5 | Pilot |
| 984 | NF - Account Provisioning | LY-NF-AP-012 User Provisioning - AD Sync process | The system shall create all accounts in the resource forest and synchronise all relevant AD Attributes between EY and the Lync AD forests Note Synchronisation with O365 will be required when UM comes into scope in a later phase Requires FIM | Pilot |
| 985 | NF - Account Provisioning | LY-NF-AP-013 User Provisioning - by office | The system shall be able to provision additional policies for a sub-set of users based on location in the HR system Requires FIM | Pilot |
| 986 | NF - Account Provisioning | LY-NF-AP-014 User Provisioning - Not permitted Lync | The system shall provide a manual override for users who are not permitted Lync based on (Country and sub-service-line? Attributes to be defined as part of design) Requires FIM | Pilot |
| 987 | NF - Account Provisioning | LY-NF-AP-015 User Provisioning - based on AD features provision policies | The system shall be able to provision additional features via policies based on attributes from AD Requires FIM | Pilot |
| 988 | NF - Account Provisioning | LY-NF-AP-016 User provisioning - Policy override | The system shall provision policies based on the following order of precedence: Country default AD attribute to override default AD Group override Requires FIM | Pilot |
| 989 | NF - Account Provisioning | LY-NF-AP-017 User provisioning - dial plans | The system shall apply a dial plan for a user based on their country in the HR system Requires FIM | Pilot |
| 990 | NF - Account Provisioning | LY-NF-AP-018 User Provisioning - Voice Policies | The system shall be able to apply a voice policy based on a pre-defined combination of AD attributes Requires FIM | Pilot |
| 991 | NF - Account Provisioning | LY-NF-AP-019 User disablement | The system shall disable the Lync account when the HR account is disabled Requires FIM | Pilot |
| 992 | NF - Account Provisioning | LY-NF-AP-020 User Deletion | The system shall delete the Lync account when the HR account is deleted Requires ATT script | Pilot |
| 993 | NF - Account Provisioning | LY-NF-AP-021 Business Card Data | The system shall populate the Business card properties in accordance with specified mapping Requires FIM | Pilot |
| 994 | NF - Account Provisioning | LY-NF-AP-022 Allocate account by country to data centre | The system shall use a mapping of countries to datacentres to decide what data centre to allocate a user to Requires FIM | Pilot |
| 995 | NF - Account Provisioning | LY-NF-AP-023 Provision to pool with least users in appropriate data centre | The system shall check which server pool in the relevant data centre has least users and allocate the user to that data centre Requires AT&T script | Pilot |
| 996 | NF - Account Provisioning | LY-NF-AP-024 Apply list of blocked dialing codes by country | The system shall be able to apply a list of blocked 'premium' dialling codes by country to voice policies Requires FIM | Pilot |
| 997 | NF - Account Provisioning | LY-NF-AP-025 Restrict any Lync voice by country | The system shall be able to restrict use of Lync voice for countries where VOIP is not permitted Requires FIM | Pilot |
| 998 | NF - Account Provisioning | LY-NF-AP-026 Restrict any Lync TEHO by country | The system shall be able to apply a policy to block PSTN rerouting based on country Requires FIM | Pilot |
| 1006 | NF - Account Provisioning | LY-NF-AP-027 Lync Tech Preview De-Activation - Automated | The system shall de-activate (set back to dormant) Lync clients without need for user intervention after Technical preview is complete | Pilot |
| 1019 | NF - Account Provisioning | LY-NF-AP-028 Lync Activation - Startup in background | The system shall start Lync in the background on system startup except the first time of use | Pilot |
| 1020 | NF - Account Provisioning | LY-NF-AP-029 Lync Activation - Splash Window first time of Launch | The System shall display a splash screen welcoming the user to Lync and containing links to learning materials Test on GCL 4 and 5 | Pilot |
| 1021 | NF - Account Provisioning | LY-NF-AP-030 Lync Activation - Open Lync Client in foreground first time only | The system shall start Lync in the foreground the first time it starts up Test on GCL 4 and 5 | Pilot |
| 1022 | NF - Account Provisioning | LY-NF-AP-031 Lync Activation - No use of Lync First Lauch package | The system shall not start the Microsoft 'First Launch' package on start up | Pilot |
| 850 | NF - Anti-Virus | LY-NF-AV-001 Optimisation of virus checking for Lync Servers | The anti-virus software shall be configured to check all Lync directories in such a way that impact on performance is minimised. | Phase 1 |
| 852 | NF - Availability - Performance - Capacity | LY-NF-AVAIL-002 24/7 global access | Users globally shall be able to access the Lync client 24/7 | Phase 1 |
| 853 | NF - Availability - Performance - Capacity | LY-NF-AVAIL-003 Prohibit the internal transfer via IM of files greater than 15MB | The System shall prohibit the internal transfer via IM of files greater than 15MB | Phase 3 |
| 854 | NF - Availability - Performance - Capacity | LY-NF-AVAIL-004 Support for minimum 176,000 Lync users | The system infrastructure shall be sized to accommodate a minimum of 176,000 Lync users | Phase 1 |
| 855 | NF - Availability - Performance - Capacity | LY-NF-AVAIL-005 99.9% availability of core Lync | The system shall maintain availability of core Lync functions to 99.9% in each data centre | Phase 1 |
| 856 | NF - Availability - Performance - Capacity | LY-NF-AVAIL-006 99.99% availbility of outbound dial | The system shall maintain availability of Outbound dial Lync functions to 99.99% in each data centre | Phase 1 |
| 857 | NF - Availability - Performance - Capacity | LY-NF-AVAIL-007 AV conferencing and Voice calls restricted if Network bandwidth is limited | The system shall restrict AV conferencing and voice calls if network bandwidth is limited | Phase 1 |
| 858 | NF - Availability - Performance - Capacity | LY-NF-AVAIL-008 Component fault tolerance for drives, network adapters, WAN links, routers, switches and hardware load balancers | The system shall include Component fault tolerance for drives, network adapters, WAN links, routers, switches and hardware load balancers | Phase 1 |
| 859 | NF - Availability - Performance - Capacity | LY-NF-AVAIL-009 Central management of address book downloads and video calls | The system shall ensure that user features that have infrastructure impact (such as bandwidth consumption for address book downloads or video calls), are centrally manageable per user or site. | Phase 1 |
| 860 | NF - Availability - Performance - Capacity | LY-NF-AVAIL-010 Dedicated Audio Video Conferencing pools in each datacenter | The system shall have Dedicated Audio Video Conferencing pools in each datacenter | Phase 1 |
| 879 | NF - Availability - Performance - Capacity | LY-NF-AVAIL-012 Support for Minimum of 50,000 Lync users in AMER region 9% per annum growth | The system infrastructure shall be sized to accommodate a minimum of 50,000 Lync users in AMER region with 9% per annum growth | Phase 1 |
| 880 | NF - Availability - Performance - Capacity | LY-NF-AVAIL-013 Support for Minimum of 90,000 Lync users in EMEIA region with 9% per annum growth | The system infrastructure shall be sized to accommodate a minimum of 90,000 Lync users in EMEIA region with 9% per annum growth | Phase 1 |
| 881 | NF - Availability - Performance - Capacity | LY-NF-AVAIL-014 Support for Minimum of 40,000 Lync users in ASIAPAC region with 9% per annum growth | The system infrastructure shall be sized to accommodate a minimum of 40,000 Lync users in ASIAPAC region with 9% per annum growth | Phase 1 |
| 861 | NF - Installation and Configuration | LY-NF-INST-001 Client Configuration identical across all data centres | The client configuration parameters shall be set to identical settings across all data centres | Pilot |
| 862 | NF - Other | LY-NF-OTH-001 SIP Domains match user SMTP/Email address | SIP Domains shall match user SMTP/Email address (for example, user01@cc.ey.com) | Pilot |

Functional specifications

This program requires installation and configuration of Commercial Products.  There will be no software development or customization.  As a result functional specification via use cases does not offer any benefits to the project and has therefore been omitted.

Design elements (completed with Application Engineering)

This program requires installation and configuration of Commercial Products.  There will be no software development and limited user interface customization.  As a result functional specification screen design elements does not offer any benefits to the project and has therefore been omitted.

Dependencies

The following dependencies exist to the specification of this feature:

No specific dependencies identified.

Risks

The following risks are associated with the specification of this feature:

No specific risks identified.

Open items

| # | Logged by | Description | Date logged | Date resolved | Resolution | Status |
| --- | --- | --- | --- | --- | --- | --- |
| 1a | Ally Ward | Co-existence between Lync and other softphone solutions such as Alcatel Lucent My Instant Communicator and Cisco IP Communicator Softphone | 03/08/2012 |  | Sandra testing MyIC & CIPC softphones | Open |
| 1b | Ally Ward | Co-existence between Lync and other webcam video solutions such as Tandberg Movi. | 4/10/12 |  | Joey Weisenstein is testing co-existence - But MOVI will be removed once tendberg integration is complete | Open |
| 2 | Ally Ward | Federation with partners other than MSFT, IBM and ATT. Lync is not designed to behave differently when communicating with federated partners than when communicating internally. This may create Q&RM issues where it could be preferable to limit some functionality communicating with federated clients | 03/08/2012 |  | Federation other than with existing partners is a phase 2 requirement. Q&RM will need to decide whether the core behaviour delivers acceptable risks or whether amendments to the behaviour of Lync need to be made before federation can be accepted | Open |
| 3 | Ally Ward | Address Book Search parameters. Need to determine which address book attributes are searched when a user instigates a search for a contact - Primary email address - Email alias - All email addresses - Company - Display name - First name - Last name | 03/08/2012 |  | With James Memmott for discussion. May need a DA decision | Open |
| 4 | Ally Ward | Sizing of data centres in terms of number of users and planning for growth. Lack of clarity for source of figures used for sizing and decision needed for how many years growth planned for | 03/08/2012 |  | Comparison being made between sametime figures, headcount figures and numbers currently in AD. Juan Torres engaged to make a decision on how many years growth to allow for. Once decided, server list may need amending | Open |
| 5 | Ally Ward | Chat archiving for Capital Advisors – Q&RM have closed this issue but there is still a requirement for this group to have archiving or they won’t benefit from the advantages Lync can offer | 03/08/2012 |  | Decision of scope to be agreed between Neil Sahdev and Juan Torres | Open |
| 6 | Ally Ward | Recording and playback of voice and video conferences | 03/08/2012 |  | Issue currently with Q&RM for discussion | Open |
| 7 | Ally Ward | Recording and playback of voice and video peer to peer communications | 03/08/2012 |  | Issue currently with Q&RM for discussion | Open |